### EECS2021 - Lab A Report

- Intro: This lab is about basic arithmetic operations as well as the assembly instructions involved with them. This includes general integer arithmetic, binary shifts, logical operations, loading larger values, and machine instructions and formats.

- Explanation:

# 0.1 a1a05.asm

This code introduces how hardware can do arithmetic at a low level. We begin by using the instruction “addi”, standing for add immediate, which adds the immediate value in the last argument to the register in the second argument, then saves that sum to the first register. In this case, we use it twice, adding the value stored in the x0 register (always 0) to the immediate number 2, and 3, and save those sums to the registers x6, and x7, respectively. Now that we have 2 and 3 saved to their respective registers, we add them together using the “add” instruction that takes the second and third argument registers, sums them, and saves it to the register in the first argument. This is the end of the activity

# 0.2 a1b05.asm

This is a shorter form of the above code, making use of the properties of the “addi” instruction, entering in the x6 register into the second argument of the second “addi” instruction so that we take the value of x6, add 3 directly to it, and save to x5 as our output. That is the entire code.

# 0.3 a1c05.asm

This is the same code as example one, but changing the 3 (immediate value) in the second “addi” instruction to -3 to demonstrate how a processor would obtain the difference. We do this by obtaining 2 and -3 in the registers x6, and x7, respectively in the ways mentioned above. Then, we add both registers together using “add” and then save it to x5, where we output our value of -1. As mentioned in the lab, the reason why the hexadecimal value shown in the registers is 0xFFFFFFFFFFFFFFFD is because -1 is 0xFFFFFFFFFFFFFFFF, and therefore, if we lower the value further by reducing the F in the 1’s slot to D, the number will in turn lower to -3.

# 0.4 a1d05.asm

This activity was the same code as shown above, but instead of adding the registers at the end, we use a different command to obtain the difference called “sub” standing for subtract. “sub” acts in the same way “add” does, but takes the second and third argument register, and subtracts them, then saves them to the first argument register. In this case, we take 2 in x6, and 3 in x7, obtain the difference using “sub” and then save the result (-1) to the register x5.

# 0.5 a2a05.asm

In this code, we are introduced to a new instruction in assembly, “slli”, standing for shift left logical immediate. This new instruction once again has a first argument that is used as its output register, and the second register in the input register. The last argument is an immediate value, that acts as the number of shifts we make. Now, shifts might not make sense, but the idea is that we take the current binary value of the second argument register, and shift all the bits left for the number of times in the immediate value, and fill in the empty spaces on the left with zeroes. This effectively is multiplication, where you can actually calculate what you are multiplying by by using 2^immediate value. So in this case, our second register gives us 17 from it being loaded in the first line using “addi”, then we use “slli” to shift the position of all the binary numbers in 17 twice. Using our formula, this means we multiply 17 by 2^2, so 17 times 4, which equals 68, and that is precisely what we get in RVS.

# 0.6 a2b05.asm

In this code, the code is near identical to the above exercise, but instead we replace “slli” with “srli” which does exactly what you might imagine, it shifts the binary value right (bits) for the amount of times in the immediate value. In this case, our second argument gives us 88, and we shift right 3 times. We can say 88 / 2^3, and when we save to register x5, we get 11 in our assembly simulator, which is correct!

# 0.7 a2c05.asm

This example showcases a cool corner case for when we shift a negative number far enough left. If the bits are moved far enough left, the entire number swaps to the beginning, and the number goes from hugely negative to hugely positive number at the far limit of the positive side of a signed integer. In this case, we start with -88 as our second register argument, and shift it left three times according to our immediate value. This forces all the bits 3 spaces left, filling in the right side of the binary number to be all zeroes, and saving the number 2305843009213693941 to the register x5. The reason the number got so large is because a number like -88 has a binary number of something like 0x11111111... and it continues with a few numbers set to 0 so that the total difference from -1 is about 87, so -1-87 is -88. When we shift all the bits right though 3 times, it moves the whole binary number 3 digits right, so we would end up with 0x00011111... The way signed integers work, they rely on the highest value digit, so the farthest left to be its sign, so 1 is negative and 0 is positive. Because we shift the 1 3 units right, the “sign” bit is now 0, so we move nearly all the way to the end of the positive side and have a very large positive signed integer.

# 0.8 a2d05.asm

This is a very similar example to activity 6, but instead we initialize the number -88 to our x6 register using the “addi” and use a new instruction “srai” which stands for shift right arithmetic immediate, to divide -88 by 2^3, and then we save it to the first argument register, so x5 is equal to -11. The reason weird things don’t happen in this case is because “srai” takes into account which sign is being used in the signed integer, so if the “sign” bit is 1 (negative) it would fill the spaces with 1’s, and vice versa for positive numbers, where it would fill regularly with 0’s.

# 0.9 a2e05.asm

This exercise consolidates out knowledge, so we initialize our first register, x6, using out “addi” method, with the immediate value 0x123. Now, we use “slli” to shift our value in register x6, 56 times left, so effectively multiply our number by 2^56, and we save this to register x7, giving us the result 0x2300000000000000 in Hexadecimal, which is, as we’ve learned from previous exercises, a very large number, near the limit of our 64 bit signed integer. Then, we shift the signed integer back 60 spots, leaving us with 0x0000000000000002, which is, a very small number, literally just the number 2. This marks the end of our learning when it comes to Binary Shifts.

# 0.10 a3a05.asm

In this code, we use a new instruction "andi" which takes a second argument register as input, and uses a "mask" to mask out bits.  For example, if we had the number 0x123, and we use 0x0f0 as our mask in the immediate value, the instruction "andi" would save 0x020 to the first argument register.  The way this works is by saying that if there is a zero in the "mask" in the immediate value, then we save those bits and they are carried over into the output, otherwise, the bits are deleted from the output.  At the end of the code, it shifts the position right twice, effectively diving the number by 2^4, and that is the end.

# 0.11 a3b05.asm

In this code, it uses several instructions that we have already used prior, so I will skip over them and move to the new thing, the instruction "or".  "or" takes 3 registers as its arguments, the first one being the output register, and the second and third as input registers.  It compares all the bits in the numbers, and if either register has bits at each individual location, the bits are carried over into the output register.  So, if I had 11001001 and 11010001 and I used "or" on these two 8-bit integers, I would get 11011001.  In this code, we use the "addi" instruction to get 2 registers with numbers, then use "andi" to mask them to get 0x020 and 0x406 in registers x6, and x7, respectively.  Then, we use or on the registers and output to register x5, which contains the value 0x426 at code completion.

# 0.12 a3c05.asm

In this code, it begins by using the "addi" instruction to get 0x123 saved to register x6.  Then, we have a new instruction called "xori", which takes 2 register and 1 immediate value as its arguments.  As usual, the first register is used as its output register and the second register is used as the input register for the instruction, and the immediate value is what to compare to.  The way "xori" works is it takes the input register, inverts it depending on the immediate value, and then returns it to the output.  For example, if I had 0x0000000000000123 and I used "xori" on it with the immediate value set to -1 (as done in the code), it would invert every bit by swapping it from 0 to 1, or vice versa, and return that to the register, so in this case, it would return 0xFFFFFFFFFFFFFEDC to register x5.

# 0.13 a4a05.asm

In this code, we use a new instruction "lui", meaning "load upper immediate" to access more storage in our 32 bit number for example.  Because of the format that, for example, "addi" uses, it can only create an integer using up to 12 bits, but lui only has 2 input arguments, allowing for more bits to be given in its input argument (up to 20 bits).  This means we can access the upper side of the 32 bit integer by using a combination of "addi" and "lui".  In this example, we used lui to load the the bits 0000 0000 0000 0000 0001 onto the register, and then add 2 using "addi", so we end up with 0000 0000 0000 0000 0001 0000 0000 0010 as the bits for our register in x6.

# 0.14 a4b05.asm

This next code literally builds off the last example, by adding on an extra "addi" and "add" to the end of the code. First, it saves 3 using "addi" to the register x6, then it uses "add" to add the register x6, which contains our binary number from earlier, and x7, which contains the 3.  When it adds them, and saves to register x5, the final result is 0000 0000 0000 0000 0001 0000 0000 0101, or 4101, which is 4096 + 5, or 4096 + 3 + 2, which is basically exactly what we did in the code.

# 0.15 a4c05.asm

This next code is the same thing as the above code except it uses a short hand version by using "addi" to directly add 3 to the register x6 and immediately save that value to x5 as our output, instead of using an external register and saving it using the "add" command.

# 0.16 a5a05.asm

In this code, it demonstrates to us how the formatting for the instructions works.  The entire code is a simple add instruction "add x5, x6, x7" but the output is actually the binary code of the format.  In this case, "add" uses R format, which contains funct7, rs2, rs1, funct3, rd, and OPCODE.  The output for the code is: 0000000 00111 00110 000 00101 0110011. OPCODE is the actual instruction/operation we are using, so in this case, the 7 bit number at the end is the OPCODE.  rd is the destination register, and that is the 5 bits to the left of OPCODE. funct3 is extra space that helps define which operation we are using, and its the following 3 bits to the left of rd. Then, rs1 and rs2 are both register slots that act as input registers, and they are 5 bits each, so the 10 bits left of funct3.  Finally, the last 7 bits is funct7, which is extra space for more operations, and that fills out all 32 bits.  And that's the whole code.

# 0.17 a5b05.asm

This code is very similar to the above code, but instead we use "addi" an instruction that uses I format.  The output is as follows: 000000000010 00000 000 00110 0010011.  The first 12 bits are for the immediate value or imm.  The next 5 bits are for the input register, or rs1.  The following 3 bits is again for funct3, and the next 5 bits are the same, rs.  Finally, the last 7 bits is the OPCODE as said before.  So the same idea, but different formatting for the bits.  This marks the end of the lab.

Conclusion

This lab was very long, and it took me a long time to write the report so that I could read it later and understand it myself.  I tried to write each explanation in simple text so that it would be easy for both me and the TAs to understand, but that led to it being very lengthy and somewhat overwhelming.  This lab was interesting though because it taught me about how the different instruction in RISC-V work, and taught me a lot about how computers deal with operations and various other important instructions.  This lab was definitely hard, but I'm glad I was able to get through it in the end.